HOME SCIENCE PAPER 2

MARKING SCHEME

|  |  | Score |  |  | Actual score |
| --- | --- | --- | --- | --- | --- |
| 1 | PRESENTATION - Work well presented (1) and well folded (½) - Label (½) firmly stitched (1) on single fabric (½) without concealing details (½) - Pins/needles (½) and unnecessary tacking threads (½) and loose threads (½) removed - Make up the right half | 1½ 2½ 1½ 1 |  |  |  |
|  |  | 6½ |  |  |  |
| 2 | CUTTING OUT - All 5 pieces cut out (5x½) - Smooth cutting of skirt CF - CF cut on straight grain to within 3 mm - Smooth cutting of yoke CF - CF yoke cut on straight grain to within 3 mm - Smooth cutting of skirt CB - CB cut on straight grain to within 3mm - Smooth cutting of the pocket pieces - Smooth cutting of the skirt at waist back (1) front (1) | 2½ 1 1 2 1 2 1 2 2 |  |  |  |
|  |  | 14½ |  |  |  |
| 3 | BACK DART - Made with straight stitchery (1) tapering to a point (½) - Thread ends well fastened at the point - Correct length of dart (9 cm) to within 5 mm - Correct width of dart (1.2 cm) to within 2 mm - Flatness of dart on WS (½) and RS (½) - Dart pressed towards C.B | 1½ 1 1 1 1 1 |  |  |  |
|  |  | 6½ |  |  |  |
| 4 | YOKE ATTACHMENT - Gathers evenly distributed - Gathers confined between the two dots - Seam made with stitchery (1) close to the fold (1) * (if not an overlaid seam give zero from c – g) - Correctly made seam i.e. yoke folded over the skirt - Raw edges well neatened (accept any correct method of neatening except pinking) - Evenness of the neatened seam allowance - Correct width of seam allowance 1 cm to within 2 mm - Yoke CF flash with skirt CF to within 2 mm - Correct shape () and size () to within 5 mm (superimpose) | 2 1 2 ½ 2 2 1 ½ 2 |  |  |  |
|  |  | 13 |  |  |  |
| 5 | POCKET - Pocket pieces attached with straight stitchery (1x2) - Notches matching (1x2) if trimmed give zero (0) - Pocket pieces appropriately placed - Pocket pieces attached 5mm away from the side seams, to within 2 mm - Smooth stitchery round the pocket bag (2) and loop stitching half the pocket bag (1) - Correct shape (1) and size (1) of the pocket (superimpose) - Pocket mouth reinforced at the base (½) and top (½) to lie on the front piece (½) - Correct width of the mouth i.e. between the dots | 2 2 2 2 3 2 1½ 1 |  |  |  |
|  |  | 15½ |  |  |  |
| 6 | SIDE SEAM – OPEN SEAM - Seam joined with straight stitchery above (1) and below (1) the pocket - Pocket mouth left unstitched - Raw edges well neatened below (1 x 2) & above (½ x 2) the pocket - Evenness of the seam allowances below (1) and above (1) the pocket - Correct size of neatened seam allowances i.e. 1 cm to within 2 mm below (1) and above (1) the pocket - Back seam allowance snipped (½ x 2) and neatened above (½) and below (½) the pocket - Seam pressed open (1) and flat (1) at the hem - Back and front skirt pieces flash at waist |  | 2 1 3 2 2 2 2 1 |  |  |
|  |  |  | 15 |  |  |
| 7 | HEM - Raw edges concealed - Evenness of the hem depth (2) and correct size 3.3 cm to within 5 mm (2) - Flatness of the hem - Quality of slip hemming - Stitches evenly distributed - Correct tension - Stitches inconspicuous on the R.S ( if not slip-hemmed give zero (0) ) - Side seam matching to within 2 mm at the hem - Hem flash at CF (1) and CB (1) |  | 2 4 2 2 2 2 2 1 2 |  |  |
|  |  |  | 19 |  |  |
|  |  |  | 90 |  |  |
|  | TOTAL = (90÷ 2) |  |  |  |  |